# 🤖 OpenCV & UGOT Robot

Commands Cheat Sheet

Your Quick Reference Guide for Computer Vision & Robot Navigation

📦 IMPORT STATEMENTS

These commands load the libraries we need:

| Command | What It Does |
| --- | --- |
| from ugot import ugot | Loads the UGOT robot library |
| import cv2 | Loads OpenCV for computer vision |
| import numpy as np | Loads NumPy for working with arrays/images |
| import time | Loads time module for delays |
| from IPython.display import Image, display | Loads display functions for Jupyter |

🎨 OPENCV COMMANDS - Color Detection

These commands help the robot "see" and understand colors:

| Command | What It Does |
| --- | --- |
| cv2.cvtColor(img, cv2.COLOR_BGR2HSV) | Converts image to HSV color space (easier to find colors!) |
| cv2.inRange(hsv, lower, upper) | Creates a mask showing only pixels within color range |
| cv2.findContours(mask, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE) | Finds the outlines of colored objects |
| cv2.boundingRect(contour) | Gets x, y, width, height of a rectangle around the object |

✏️ OPENCV COMMANDS - Drawing

These commands draw shapes and text on images:

| Command | What It Does |
| --- | --- |
| cv2.rectangle(img, (x1,y1), (x2,y2), color, thickness) | Draws a rectangle (box) around objects |
| cv2.circle(img, (cx,cy), radius, color, thickness) | Draws a circle at a point (-1 = filled) |
| cv2.line(img, (x1,y1), (x2,y2), color, thickness) | Draws a straight line |
| cv2.putText(img, text, (x,y), font, scale, color, thickness) | Adds text message to the image |

🖼️ OPENCV COMMANDS - Image Processing

| Command | What It Does |
| --- | --- |
| cv2.imencode('.jpg', img) | Converts image to JPEG format for display |
| cv2.imdecode(nparr, cv2.IMREAD_COLOR) | Converts camera data back to an image |
| img.copy() | Makes a copy of the image (protects original) |

🔢 NUMPY COMMANDS

NumPy helps us work with image data as numbers:

| Command | What It Does |
| --- | --- |
| np.array([0, 100, 100]) | Creates an array (list of numbers) for HSV values |
| np.zeros((height, width, 3), dtype=np.uint8) | Creates a blank black image |
| np.frombuffer(frame, np.uint8) | Converts camera data to NumPy array |

🤖 UGOT ROBOT COMMANDS

These commands control your UGOT robot:

| Command | What It Does |
| --- | --- |
| got.open_camera() | Turns on the robot's camera |
| got.read_camera_data() | Takes a picture from the camera |
| got.mecanum_move_xyz(x, y, z) | Moves the robot: x=left/right, y=forward/back, z=rotation |
| got.mecanum_stop() | Stops all wheel movement |

🎮 Movement Examples:

| Command | Direction | Robot Movement |
| --- | --- | --- |
| mecanum_move_xyz(0, 10, 0) | Forward | Robot moves straight ahead |
| mecanum_move_xyz(5, 5, 0) | Forward-Right | Robot moves diagonally right |
| mecanum_move_xyz(-5, 5, 0) | Forward-Left | Robot moves diagonally left |
| mecanum_move_xyz(10, 0, 0) | Strafe Right | Robot slides sideways right |
| mecanum_move_xyz(0, 0, 10) | Rotate Right | Robot spins clockwise |

🌈 HSV COLOR REFERENCE

Common color ranges for detection (Hue, Saturation, Value):

| Color | Lower Range | Upper Range |
| --- | --- | --- |
| RED | [0, 100, 100] | [10, 255, 255] |
| GREEN | [35, 100, 100] | [85, 255, 255] |
| BLUE | [100, 100, 100] | [130, 255, 255] |
| YELLOW | [20, 100, 100] | [35, 255, 255] |
| ORANGE | [10, 100, 100] | [20, 255, 255] |

💡 QUICK TIPS

- Always use img.copy() before drawing on images to keep the original safe!

- Colors in OpenCV are BGR (Blue, Green, Red), not RGB!

- Use time.sleep(0.1) to prevent the loop from running too fast!

- The center of the camera image is usually around x=280 for a 560-wide image.

- Larger bounding box height = object is closer to the robot!

🎓 Happy Coding with OpenCV and UGOT Robots! 🎓